# GPS Hardware Options for the Firebird Multi-Gauge

A running record of the GPS-module-and-antenna decisions for the Firebird gauge, captured here so we don’t have to re-derive it next time we’re picking parts. Updated as new options come up.

# Current state on the bench

Working: Beitian BN-880 GPS module, powered externally at 5 V (NOT off the Waveshare’s JST 3 V3 — the BN-880 needs 3.6 V minimum, which the JST can’t supply). UART1 on GPIO 43/44 reads NMEA at 9600 baud. Reaches TRACKING 10 satellites in the garage; gets a real FIX outdoors or by a window.

The firmware is solid; the limitation is purely physical — the internal patch antenna on the BN-880 needs a clear sky view, which isn’t going to happen behind a metal-roof car dashboard.

# Constraints we’re shopping against

- Module body must be hideable. Going behind the center dash speaker grille if reception there allows it; otherwise tucked inside the gauge enclosure.

- Antenna must be a small black hockey-puck on a magnetic / adhesive base with ~3 ft of cable, mountable on the dashboard with a clear view through the windshield.

- Output: 9600 baud NMEA over UART, 3.3 V or 5 V TTL. That’s what the firmware in main/GPS/GPS.c expects.

- Power: ideally runs off the Waveshare’s JST 3 V3 pin so we don’t have to add a buck converter. If the module needs 5 V, it has to tap into the upstream 12 V → 5 V regulator that powers the Waveshare itself.

- Antenna connector: SMA jack on the module (or u.FL with adapter) so any standard puck antenna screws on.

# Options we’ve considered

## Option A — Keep the BN-880, add external 5 V supply (current setup)

| Item | Cost | Notes |
| --- | --- | --- |
| Existing BN-880 | $0 | already on hand |
| 12 V → 5 V buck converter | ~$3–5 | needed for car install |
| External puck antenna | n/a | Jeff confirmed this BN-880 has no u.FL |

Verdict: Works on the bench but doesn’t solve the hide-the-module problem because BN-880’s antenna is internal. Not the long-term answer. Useful as the proof-of-concept while waiting for a better module to ship.

## Option B — NEO-7M module with built-in SMA jack + separate puck antenna (RECOMMENDED)

| Item | Cost | Notes |
| --- | --- | --- |
| NEO-7M GPS Module with SMA Antenna Interface, 3.3 V/5 V TTL | ~$10 | replaces NEO-6M, well-known chip |
| Geekstory Active GPS+GLONASS Antenna, 33 dB, 3 m cable, IP67 | ~$15 | black puck, magnetic, SMA male |
| Total | ~$25 |  |

Pros:

- Runs on 3.3 V off the Waveshare JST — no buck converter needed.

- Default 9600 baud NMEA — drop-in firmware compatibility, no changes to main/GPS/GPS.c.

- SMA jack is industry-standard, antenna swappable forever.

- 33 dB gain on the antenna is strong enough to punch through the dash speaker grille.

Cons:

- NEO-7M is GPS-only (no GLONASS/BeiDou). Doesn’t matter for North American driving; plenty of GPS satellites overhead at all times.

- Older chip (~2013) than the M8N. Slightly slower cold-start TTFF (~30 s vs ~25 s). Not noticeable in practice.

This is the lowest-risk, lowest-cost path. If you want to spend money once and forget it, buy these two parts.

## Option C — ABUQ NEO-M8N module with SMA jack + separate puck antenna

| Item | Cost | Notes |
| --- | --- | --- |
| ABUQ NEW GPS Mini Module Neo-8n NEO-M8N-0-01 with SMA Head | ~$15 | does NOT come with antenna |
| Geekstory 33 dB puck antenna (same as Option B) | ~$15 | black puck, magnetic, SMA male |
| Total | ~$30 |  |

Pros:

- NEO-M8N is multi-constellation (GPS + GLONASS + BeiDou), 72 channels.

- Slightly faster TTFF and better multipath rejection than the 7M.

- SMA jack on the module.

Cons:

- Default baud rate not confirmed. Could ship at 9600, 38400, 57600, or 115200. Most “Arduino-targeted” listings are 9600 NMEA but ABUQ’s listing didn’t specify.

- Output protocol not confirmed. Most ship as NMEA, but some Pixhawk-targeted M8N modules default to UBX binary.

Firmware mitigation if it ships at the wrong baud / protocol:

- I’ll add an auto-probe to GPS_Init() that tries 9600 → 38400 → 57600 → 115200 in sequence, locking onto whichever yields valid NMEA. ~30 lines of new code, runs once on boot, +5 s startup time worst case.

- If the module ships in UBX binary mode, I’ll add a one-time configuration sequence (CFG-PRT, CFG-MSG, CFG-CFG) that flips it to NMEA at 9600 and saves the config to its flash so it sticks. ~80 lines of code.

Verdict: Not zero-risk, but recoverable if it ships funky. Pick this if you specifically want the M8N chip / multi-constellation.

## Option D — Beitian BN-880Q (alternative cheap drop-in)

| Item | Cost | Notes |
| --- | --- | --- |
| Beitian BN-880Q GLONASS GPS Module | ~$15 | NEO-M8N chip, 3.0 V–5.5 V range |

Pros:

- Wider voltage range (3.0–5.5 V) so it’ll run off the JST 3 V3 pin unlike our current BN-880. No external 5 V supply needed.

- 9600 baud NMEA default.

- NEO-M8N chip.

Cons:

- Internal patch antenna only (or u.FL — varies by batch). Doesn’t solve the hide-the-module problem unless we add an external antenna separately, in which case we’re back at Option C.

Verdict: Useful if we just want to fix the 5 V power issue and keep the same form factor, but doesn’t get us to a hidden install.

## Option E — SparkFun NEO-M9N SMA breakout (premium)

| Item | Cost | Notes |
| --- | --- | --- |
| SparkFun NEO-M9N (Qwiic) with SMA jack | ~$50 | newest u-blox chip |
| Geekstory 33 dB puck antenna | ~$15 |  |
| Total | ~$65 |  |

Pros:

- Newest u-blox chip (M9N, post-M8). Best TTFF and multipath in the family.

- SparkFun documentation, libraries, and warranty.

Cons:

- Overkill for displaying MPH on a Firebird gauge.

- 2x the cost of Option B with no real benefit for this use case.

Verdict: Unnecessary. Skip.

## Option F — Original BN-880 + u.FL pigtail + puck antenna (DEAD END)

I kept suggesting this because most BN-880 modules do have a u.FL connector for an external antenna. Jeff confirmed his specific module does NOT have u.FL accessible. Removed from consideration.

# Recommendation

Buy Option B (NEO-7M + Geekstory puck antenna, ~$25). Lowest cost, lowest risk, runs on the JST 3 V3 with no buck converter, and the firmware needs no changes whatsoever. The “older chip” caveat is academic — you’ll never notice the TTFF difference on a Firebird gauge.

If for some reason Option B doesn’t pan out (out of stock, looks weird, etc.), Option C (ABUQ NEO-M8N + same puck antenna) is the next choice with the auto-probe firmware mitigation.

# Antenna positioning on the dashboard

Once the puck antenna arrives, install plan:

- Module body: tucked behind the center dash speaker grille, or inside the gauge enclosure if grille reception is too weak.

- Antenna puck: stuck (magnetic + adhesive) on the top of the dashboard near the windshield, label / sky-side facing UP. Short SMA cable run from antenna to module body, hidden under the dashpad.

- Power: 3 V3 from the JST UART connector on the Waveshare board. GND tied to the same JST GND.

- Data: GPS-TX (green) → JST RXD; GPS-RX (white) → JST TXD.

- Firmware: nothing to change. Flash, reset, watch the speed gauge’s footer go through NO FIX → TRACKING N → FIX N SATS as the antenna locks onto satellites.

# What’s still unknown

- Whether reception through the center speaker grille is good enough. The grille has a metal speaker frame behind it that may attenuate signal. Tomorrow’s outdoor test with the BN-880 in that location will tell us. If yes, the puck antenna can sit on the top of the dashpad inside the gauge enclosure. If no, the puck has to mount on a clear sky-view spot (windshield base, dashpad top, etc.).

- Whether the NEO-7M ships with the SMA antenna pre-attached (the small ceramic patch most NEO-7M kits include). Doesn’t matter — we’d swap it for the puck regardless, just affects what’s in the box.